Understood! You want to build an **ESP32-based drone (without Arduino)** for **pothole detection**, using the **ESP32-CAM** for image capture and streaming directly to a smartphone app. Here’s a streamlined approach using **ESP-IDF (Espressif’s native framework)** and **raw ESP32 capabilities**:

---

### **System Overview**

1. **Hardware**:

- **ESP32 (Main Controller)**: Runs flight control (PID, motor drivers).

- **ESP32-CAM**: Captures images, runs pothole detection, streams video.

- **Optional**: GPS (for tagging pothole locations), ultrasonic sensor (altitude control).

2. **Software**:

- **ESP-IDF** for firmware (no Arduino dependencies).

- **FreeRTOS** for multitasking (flight control + camera).

- **Smartphone App**: Custom Android/iOS app to view stream + alerts.

---

### **Step 1: ESP32-CAM Setup (ESP-IDF)**

#### **1. Configure Camera in ESP-IDF**

- Use the **esp32-camera** component (official Espressif repo).

- Example `CMakeLists.txt`:

```cmake

idf_component_register(

SRCS "main.c"

INCLUDE_DIRS "."

REQUIRES esp32-camera

)

```

#### **2. Initialize Camera (OV2640)**

```c

#include "esp_camera.h"

void setup_camera() {

camera_config_t config = {

.pin_pwdn = 32,

.pin_reset = -1,

.pin_xclk = 0,

.pin_sscb_sda = 26,

.pin_sscb_scl = 27,

.pin_d7 = 35,

.pin_d6 = 34,

.pin_d5 = 39,

.pin_d4 = 36,

.pin_d3 = 21,

.pin_d2 = 19,

.pin_d1 = 18,

.pin_d0 = 5,

.pin_vsync = 25,

.pin_href = 23,

.pin_pclk = 22,

.xclk_freq_hz = 20000000,

.ledc_timer = LEDC_TIMER_0,

.ledc_channel = LEDC_CHANNEL_0,

.pixel_format = PIXFORMAT_JPEG,

.frame_size = FRAMESIZE_SVGA,

.jpeg_quality = 12,

.fb_count = 1

};

esp_err_t err = esp_camera_init(&config);

if (err != ESP_OK) {

ESP_LOGE(TAG, "Camera init failed: %s", esp_err_to_name(err));

}

}

```

---

### **Step 2: Pothole Detection (On-Device)**

#### **1. Simple Edge Detection (Canny Algorithm)**

- Use lightweight image processing (no OpenCV):

```c

#include "esp_dsp.h"  // For DSP-accelerated operations

void detect_potholes(camera_fb_t *fb) {

// Convert JPEG to grayscale (if needed)

uint8_t *gray = (uint8_t *)malloc(fb->width * fb->height);

jpeg_to_grayscale(fb->buf, fb->len, gray);

// Edge detection (simplified)

for (int y = 1; y < fb->height - 1; y++) {

for (int x = 1; x < fb->width - 1; x++) {

int gx = gray[(y-1)*fb->width + (x+1)] - gray[(y-1)*fb->width + (x-1)]

+ 2*gray[y*fb->width + (x+1)] - 2*gray[y*fb->width + (x-1)]

+ gray[(y+1)*fb->width + (x+1)] - gray[(y+1)*fb->width + (x-1)];

int gy = gray[(y+1)*fb->width + (x-1)] - gray[(y-1)*fb->width + (x-1)]

+ 2*gray[(y+1)*fb->width + x] - 2*gray[(y-1)*fb->width + x]

+ gray[(y+1)*fb->width + (x+1)] - gray[(y-1)*fb->width + (x+1)];

int edge = sqrt(gx*gx + gy*gy) > THRESHOLD ? 255 : 0;

// Mark edges (or potholes)

}

}

free(gray);

}

```

#### **2. Send Detected Potholes to Smartphone**

- **Option A: HTTP Server (Live Stream + Alerts)**

```c

#include "esp_http_server.h"

httpd_handle_t start_webserver() {

httpd_config_t config = HTTPD_DEFAULT_CONFIG();

httpd_handle_t server = NULL;

httpd_start(&server, &config);

// Add endpoint for JPEG stream

httpd_uri_t stream_uri = {

.uri = "/stream",

.method = HTTP_GET,

.handler = stream_handler,

};

httpd_register_uri_handler(server, &stream_uri);

return server;

}

```

---

### **Step 3: Smartphone App Integration**

#### **1. Android App (Kotlin)**

- **Live Streaming**: Use `ExoPlayer` for MJPEG stream:

```kotlin

val player = ExoPlayer.Builder(context).build()

val dataSource = ProgressiveMediaSource.Factory(

DefaultHttpDataSource.Factory()

).createMediaSource(MediaItem.fromUri("http://DRONE_IP/stream"))

player.setMediaSource(dataSource)

player.prepare()

```

#### **2. Pothole Alerts**

- Receive HTTP POSTs with pothole coordinates/images:

```kotlin

val client = OkHttpClient()

val request = Request.Builder().url("http://DRONE_IP/alert").build()

client.newCall(request).enqueue(object : Callback {

override fun onResponse(call: Call, response: Response) {

val image = BitmapFactory.decodeStream(response.body.byteStream())

runOnUiThread { imageView.setImageBitmap(image) }

}

})

```

---

### **Step 4: Flight Control (FreeRTOS Tasks)**

- **Task 1**: Flight stabilization (PID loop for motors).

- **Task 2**: Camera capture + pothole detection.

- **Task 3**: Wi-Fi communication (HTTP/MQTT).

```c

void flight_control_task(void *pvParameters) {

while (1) {

read_mpu6050();  // Get IMU data

pid_adjust_motors();

vTaskDelay(10 / portTICK_PERIOD_MS);

}

}

void camera_task(void *pvParameters) {

while (1) {

camera_fb_t *fb = esp_camera_fb_get();

detect_potholes(fb);

esp_camera_fb_return(fb);

vTaskDelay(100 / portTICK_PERIOD_MS);

}

}

```

---

### **Optimizations**

1. **Power**: Lower CPU freq (`esp_pm_configure()`) when idle.

2. **Latency**: Use **UDP** instead of HTTP for faster alerts.

3. **Detection**: Quantize ML model (TensorFlow Lite) for ESP32.

---

### **Full Code Repo**

For a complete ESP-IDF project, check:

- [ESP32-CAM Pothole Detection](https://github.com/espressif/esp32-camera)

- [ESP-IDF HTTP Server Example](https://github.com/espressif/esp-idf/tree/master/examples/protocols/http_server)

Would you like help with PCB design or motor control code? 🛠️